| Reg. No.: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

K. RAMAKRISHNAN COLLEGE OF ENGINEERING

(AUTONOMOUS)

Question Paper Code: QP123

Question Paper Code: QP123

Model Examination

Second Semester

CSE

CS3401 – Model Examination

(Regulation 2023)

| Time: Three Hours | Maximum Marks: 100 Marks |
| --- | --- |

PART- A                                                                (10 x 2 = 20 Marks)

| Q.No. | Answer ALL Questions | CO | BTL | Marks |
| --- | --- | --- | --- | --- |
| 1 | The property of the material which allows it to be drawn into smaller section? Justify your answer. a) Ductility b) Plasticity c) Elasticity d) Malleability | CO1 | BTL4 | 2 |
| 2 | A square steel rod 20mmX20mm in section is to carry an axial load (compressive) of 1000KN. Calculate the shortening in a length of 50mm. E=2.14X10⁸KN/m². | CO1 | BTL3 | 2 |
| 3 | False | CO2 | BTL3 | 2 |
| 4 | Organize collinear and concurrent force system | CO2 | BTL3 | 2 |
| 5 | Which of the following statements is true for a rigid body undergoing pure translational motion?, Justify a) The body is rotating about an axis passing through its center of mass. b) Different points on the body have different velocities. c) Every point on the body has the same velocity and acceleration. d) The linear velocity of the center of mass is zero. | CO3 | BTL4 | 2 |
| 6 | Distingush the motion curve with rectilinear motion. . | CO3 | BTL4 | 2 |
| 7 | If an I-beam is loaded such that it experiences a high shear force (V), where is the largest percentage of the total shear force resisted by the cross-section? Justify your answer. a) Uniformly across the entire cross-section. b) In the flanges (top and bottom horizontal parts). c) At the junction between the web and the flanges. d) In the web (central vertical part). | CO4 | BTL4 | 2 |
| 8 | Analyze the basic elements of truss and their types. | CO4 | BTL4 | 2 |
| 9 | Power transmitted by a shaft is given by. Justify your answer. a) P = 2πNT/60 b) P = T/2πN c) P = TN/60 d) P = 2πT/N | CO5 | BTL4 | 2 |
| 10 | Derive the torsion equation of a shaft in pump and explain each term. | CO5 | BTL4 | 2 |

PART – B                          (5 x 16 = 80 Marks)

| Q.No. | Question | CO | BTL | Marks |
| --- | --- | --- | --- | --- |
| 11.a | A steel rod of 2 cm diameter is enclosed centrally in a hollow copper tube of external diameter 4 cm and internal diameter of 3.5 cm. The composite bar is then subjected to an axial pull of 50000 N. 1f the length of each bar is equal to 20 cm, determine : i. The stress in the rod and tube, and ii Load carried by each bar. Take E for steel=2 x10^5 N/Sq mm and for copper=1 x10^5 N/Sq mm | CO1 | BTL2 | 16 |
| (OR) |  |  |  |  |
| 11.b | The following data release to a bar subjected to a tensile test. Diamter of the bar=30mm Tensile load=54kN Gauge length=300mm Extension of the bar=0.112mm Change in diamter=0.00366mm Calculate i) Poisson's ratio, ii)The values of three modulii | CO1 | BTL2 | 16 |
| 12.a | Application problems | CO2 | BTL3 | 16 |
| (OR) |  |  |  |  |
| 12.b | Real time application problems | CO2 | BTL3 | 16 |
| 13.a | Two trains A and B leave the same station on parallel lines. Train A starts with an uniform acceleration of 0.5 m/s² and attains a speed of 36 km/hour, when steam is reduced to keep the speed constant. Train B leaves 30 seconds after, with uniform acceleration of 1 m/s² to attain a maximum speed of 54 km/hour. Identify the time, the train A when will overtake to A? | CO3 | BTL3 | 16 |
| (OR) |  |  |  |  |
| 13.b | A particle is projected with an initial velocity of 10 m/s at an angle α with the horizontal. After some time, the position of the particle is observed by its x and y distances of 12 m and 8 m respectively from the point of projection. Identify the angle of projection. | CO3 | BTL3 | 16 |
| 14.a | Calculation | CO4 | BTL3 | 16 |
| (OR) |  |  |  |  |
| 14.b | Modification of existing designs and justify | CO4 | BTL3 | 16 |
| 15.a | A beam of length 6m is simply supported at the ends and carries a UDL of 1.5kN/m run and three concentrated loads of 1kN, 2kN and 3kN acting at a distance of 1.5m, 3m and 4.5m respectively from left end. Draw the SFD and BMD and determine the maximum bending moment. | CO5 | BTL3 | 16 |
| (OR) |  |  |  |  |
| 15.b | A hollow circular shaft of diameter ratio 3/8 (internal diameter to outer diameter) is to transmit 375 kW at 100 r. p. m. The maximum torque being 20% greater than mean. The shear stress is should not exceed 60 N/mm² and twist in a shaft length of 4 m not to exceed 2º. Take modulus of rigidity 0.85x10⁵ N/mm². Calculate the internal diameter and external diameter which would satisfy both the above conditions. | CO5 | BTL3 | 16 |

******************

QP123